Camera Pose Estimation Tool

Handoff & User Documentation

Version 1.0 — May 2026

# 1. Introduction

This tool is an interactive simulator for camera pose estimation developed as part of a computer vision project. It estimates the relative pose (rotation R and translation t) between two cameras from 2D point correspondences, following the methodology used in ORB-SLAM.

## What the tool does

From an automatically generated synthetic 3D scene, the tool:

- Projects 3D points onto both camera image planes.

- Estimates the pose of camera 2 relative to camera 1 using two methods:

  - Homography matrix H — suited for planar scenes (points lying on a common plane).

  - Fundamental matrix F — suited for general (non-planar) 3D scenes.

- Automatically selects the most reliable method using an ORB-SLAM-inspired score and ratio.

- Displays results in an interactive 3D view and measures rotation/translation errors against ground truth.

# 2. Prerequisites & Installation

## Python version

Python 3.9 or higher is required. Python 3.10–3.12 is recommended for best compatibility with all dependencies.

## Installing dependencies

From the project root directory, run:

pip install -r requirements.txt

## Launching the interface

From the project root:

python pipeline/gui_tool.py

Note: If using Anaconda, ensure the active environment has all packages installed. Python 3.13+ may introduce compatibility issues with some dependencies — using a dedicated virtual environment (conda create -n pose python=3.11) is recommended.

# 3. User Interface Guide

## General layout

The interface is divided into two main areas:

- Left column — a scrollable sidebar containing 4 parameter tabs and action buttons at the bottom.

- Right area — a matplotlib canvas that displays results after each Run. Before the first run, it shows a placeholder: "Configure parameters and click Run".

## Tab — Scene

Scene type

A segmented button at the top lets you choose between three scene types:

Planar — all 3D points are generated at the same depth Z (a single front-parallel plane). This is the ideal case for homography estimation: H is exactly valid for coplanar points.

- Depth Z (m) — the single depth at which all points are placed. Typical value: 5.0 m.

Non-planar — 3D points are distributed randomly across a volume. Depths vary between Z min and Z max. This is the ideal case for the fundamental matrix F.

- Z min (m) / Z max (m) — depth range of the generated points.

Urban — a fixed synthetic urban environment made of three geometric objects:

  - Building — rectangular box (3 m × 3 m × 6 m height) placed at Z = 10 m.

  - Tree — a thin cylindrical trunk topped by a hemispherical dome (mushroom shape), placed at

Z = 8 m on the left.

  - Car — two stacked boxes (body + roof), placed at Z = 7 m on the right.

The geometry is fixed; only the number of points varies.

Point cloud section

These parameters control the spatial distribution of generated points.

| Parameter | Applies to | Description |
| --- | --- | --- |
| Range X ±(m) | Planar, Non-planar | Horizontal extent of the scene in meters either side of the central axis. |
| Range Y ±(m) | Planar, Non-planar | Same principle on the vertical axis. |
| Number of points | All | Total number of 3D points generated. Range: 8 to 500. In Urban mode, points are split across the three objects: 55% building, 25% tree, 20% car. |

⚠ In Urban mode, fewer than 100 points makes the shapes hard to recognise visually. Use 300–400 points for a clear view of the scene.

Signal degradation section

These parameters simulate imperfect acquisition conditions.

| Parameter | Description |
| --- | --- |
| Pixel noise σ (px) | Zero-mean Gaussian noise with standard deviation σ added to the 2D pixel coordinates of both cameras. σ = 0.0: ideal conditions. σ = 1.0: realistic precision for a good keypoint detector. σ = 3.0–5.0: heavy degradation. |
| Outliers (%) [cam 2] | Percentage of erroneous correspondences injected into camera 2 only. An outlier replaces a projected point's pixel position with a random position anywhere in the image — simulating false keypoint matches. 0%: no outliers. >0% of matches are false (RANSAC should handle these). |

Random seed section (Planar and Non-planar only)

- "New random scene at each Run" switch — when OFF (default): the same seed is used every Run, so the scene is identical across runs (useful for comparing H and F on exactly the same data). When ON: a new random seed between 0 and 99 999 is drawn at each Run.

- Seed field — the numeric seed value (default: 42). Can be edited manually to reproduce a specific scene. When the random switch is ON, this field updates automatically after each Run — useful for noting an interesting scene and reproducing it later.

## Tab — Cameras

This tab defines the pose of both cameras in the world frame. The rotation is built in the order Rz · Ry · Rx (extrinsic Euler angles).

| Camera | Parameter | Default | Description |
| --- | --- | --- | --- |
| Camera 1 (reference) | rx ry rz (deg) | 0 / 0 / 0 | Zero rotation: camera 1 sits at the world origin looking in the +Z direction. In practice, Camera 1 is never modified — it defines the reference frame. |
| Camera 1 (reference) | tx ty tz (m) | 0 / 0 / 0 | Zero translation: the world origin. |
| Camera 2 | ry (deg) | 8° | 8° rotation around Y: slight horizontal pan of camera 2 relative to camera 1. |
| Camera 2 | tx (m) | 0.4 m | 0.4 m offset along X. These default values give a realistic stereo setup. |

## Tab — Optics

This tab defines the intrinsic parameters shared by both cameras.

| Parameter | Default | Description |
| --- | --- | --- |
| fx (px) | 1000 | Horizontal focal length in pixels. Defines the horizontal magnification of the camera. |
| fy (px) | 1000 | Vertical focal length in pixels. |
| cx (px) | 960 | Horizontal principal point (optical centre). Default = W/2. |
| cy (px) | 540 | Vertical principal point. Default = H/2. |
| Width W (px) | 1920 | Image width in pixels. Points projected outside [0, W] × [0, H] are discarded as not visible. |
| Height H (px) | 1080 | Image height in pixels. |
| FOV (auto) | ≈ 53.1° × 31.0° | Horizontal and vertical field of view, computed in real time: FOV_h = 2·arctan(W/2fx), FOV_v = 2·arctan(H/2fy). |

## Tab — Analysis

Methods section

- Homography H switch — enables or disables the homography estimation branch.

- Fundamental matrix F (8 pts) switch — enables or disables the fundamental matrix branch. At least one of the two switches must be ON to launch a Run.

Analysis mode

- Single scene — default mode. Runs the full pipeline once with the current parameters. Displays the 3D scene view, reprojection overlays, and the results card.

- Noise sweep — repeats the analysis over multiple noise levels σ defined in the "σ levels" field (default: 0  0.5  1  2  3  5, space-separated). Produces two error curves — rotation error and translation error — as a function of σ, for both H and F. Useful for comparing robustness against noise.

Save figure switch

When enabled, the matplotlib figure is automatically exported to pipeline/exports/ after each Run. The filename includes a timestamp (e.g. result_single_20260507_174532.png).

Results card

After each Run in Single scene mode, a summary box at the bottom of the Analysis tab displays:

Visible points: 115 H  →  Err R = 0.703°  |  Err t = 7.859° F  →  Err R = 0.000°  |  Err t = 0.000° Ratio = 0.472  →  [H]

- Visible points — number of 3D points simultaneously visible to both cameras (i.e. projecting within the image bounds of both).

- Err R — angular rotation error in degrees. 0° = perfect recovery.

- Err t — angular direction error of the translation vector in degrees. Note: the scale of t is not recoverable in monocular estimation, so only the direction is evaluated.

- Ratio = S_H / (S_H + S_F). Decision threshold: if Ratio > 0.45, the homography model is selected (shown as [H]). Otherwise the fundamental matrix is used ([F]).

## Action buttons (bottom bar)

- Run — launches the analysis with the current parameters. During computation the button displays "Running…" and is disabled. A progress indicator appears.

- Reset — resets all parameters to their default values and clears the display back to the placeholder.

Status bar — displays real-time feedback: "Ready.", "Analysis running…", "Analysis done · 115 pts · [F]", or error messages if something goes wrong.

# 4. Understanding the Results

## 4.1  Layout of the results area

After a Run in Single scene mode, the right-hand canvas is divided into 5 panels:

- 3D scene view

- Camera 1 – image

- Camera 2 – reprojection H

- Camera 2 – reprojection F

- ORB-SLAM score bar

## 4.2  The 3D scene view

Point cloud colouring

- Planar / Non-planar — points are coloured by depth Z using a plasma colormap (purple = near, yellow = far).

- Urban — points are coloured by object type:

  - Blue = Building

  - Green = Tree

  - Orange = Car

Camera frustums

Each camera is represented by a truncated pyramid (frustum) delimiting its field of view, plus an arrow indicating its viewing direction.

| Frustum colour | Camera | Meaning |
| --- | --- | --- |
| Blue | Camera 1 | Reference camera — fixed ground truth pose. |
| Green | Camera 2 | Ground truth pose of camera 2. |
| Orange | Camera 2 | Pose estimated via H. |
| Red | Camera 2 | Pose estimated via F. |

The winner (selected model) is drawn with a thick solid line. The loser is drawn with a thin dashed line. The closer the estimated frustum (orange or red) overlaps the green frustum, the more accurate the estimation is.

## 4.3  Reprojection panels: H and F

Principle

Reprojection consists of taking the ground truth 3D points, projecting them into camera 2 using the estimated pose (R, t), and comparing those projected positions with the actual observed 2D positions.

| Panel | Points shown | How to read it |
| --- | --- | --- |
| Camera 1 — image plane | Blue dots = ground truth projections in camera 1. | Reference view. Shows the spatial distribution of correspondences in the first image. |
| Camera 2 — Reprojection H | Green dots = ground truth positions. Orange = positions reprojected via H pose. | If it overlaps the green dots exactly, H has correctly estimated the pose. Title shows [WINNER] if H won. |
| Camera 2 — Reprojection F | Green dots = ground truth positions. Red = positions reprojected via F pose. | If it overlaps the green dots exactly, F has correctly estimated the pose. Title shows [WINNER] if F won. |

Interpreting the overlap quality

| Visual result | Interpretation |
| --- | --- |
| Perfect overlap | Pose estimation is excellent — error close to 0°. |
| Uniform offset | Rotation is well estimated but translation has a small error. |
| Scattered points | Pose is poorly estimated — too much noise, too many outliers, or wrong model selected. |

## 4.4  ORB-SLAM score and ratio

Score H — symmetric transfer error

For each correspondence (px1, px2), the transfer error is measured in both directions:

- Forward — project px1 via H into image 2, measure pixel distance to px2.

- Backward — project px2 via H⁻¹ into image 1, measure pixel distance to px1.

S_H = Σ  max(0, 5.99 − d²_forward) + max(0, 5.99 − d²_backward)

The threshold 5.99 is the 95th percentile of a chi-squared distribution with 2 degrees of freedom (2D transfer error in the image plane).

Score F — Sampson distance

The Sampson distance measures how much each point pair violates the epipolar constraint x2ᵀFx1 = 0:

d²_sampson = (x2ᵀFx1)² / (‖Fx1‖²_top2 + ‖Fᵀx2‖²_top2) S_F = Σ  max(0, 3.84 − d²_sampson)

The threshold 3.84 is the 95th percentile of a chi-squared distribution with 1 degree of freedom (the epipolar constraint is a scalar 1D constraint).

Ratio and decision

ratio = S_H / (S_H + S_F) if ratio > 0.45  →  H selected   (scene sufficiently planar) if ratio ≤ 0.45  →  F selected

Why 0.45 and not 0.50? F has more degrees of freedom and can fit data well even in planar scenes. The asymmetric threshold deliberately gives H a slight advantage, since H is the correct model for planar scenes and should win even when F is competitive.

## 4.5  Rotation and translation errors

Rotation error (°)

Measures the minimal angle between the estimated rotation matrix and the ground truth:

err_R = arccos( (trace(R_est.T @ R_ref) − 1) / 2 )

Principle: any 3D rotation can be written as a rotation by angle θ around an axis (Euler's theorem). The trace of the relative rotation matrix gives this angle directly via trace = 1 + 2·cos(θ).

| Value | Interpretation |
| --- | --- |
| 0.000° | Perfect estimation. |
| < 1° | Very good estimation. |
| 1° to 5° | Acceptable estimation. |
| > 5° | Degraded estimation. |

Translation error (°)

Why degrees, not metres? The scale of the translation vector is not recoverable without metric ground truth. Only the direction of t can be evaluated:

u = t_est / ‖t_est‖        (unit vector of estimated direction)

v = t_ref / ‖t_ref‖        (unit vector of true direction)

err_t = arccos( |u · v| )

The absolute value |u · v| handles the sign ambiguity. + t and −t point in opposite directions but share the same |cos|.

| Value | Interpretation |
| --- | --- |
| 0.000° | Direction perfectly estimated. |
| < 5° | Very good directional estimation. |
| 5° to 15° | Acceptable estimation. |
| > 15° | Direction poorly estimated. |

# 5. Code Architecture

## 5.1  Project structure

camera-pose-estimation/
│
├── pipeline/              ← Orchestration and GUI
│   ├── core.py
│   ├── gui_tool.py
│   ├── visualize3d.py
│   └── score.py
│
├── simulation/            ← Camera parameters + homography
│   ├── camera_model.py
│   ├── projection.py
│   ├── homography.py
│   ├── dlt.py
│   └── scene_urban.py
│
└── eight_points/          ← 8-point
    ├── eight_point_agl.py
    ├── RANSAC.py
    ├── Retrieve_P.py
    └── triangulation.py

## 5.2  Execution flow

The following sequence is triggered when Run is pressed:

Click Run  (gui_tool.py)
    │
    ▼
make_scene(cfg)                        ← core.py
    │
    ├── Urban     → generate_urban_scene()     ← scene_urban.py
    ├── Planar    → fixed Z, random XY
    └── Non-planar → random XYZ in range
    │
    ▼
project_points()  +  filter_visible()         ← projection.py
(+ noise injection  +  outlier injection)
    │
    ▼
run_pipeline(scene, cfg)               ← core.py
    │
    ├── Branch H ─────────────────────────────────────────────────────
    │   RANSAC(s=4, score_H_RANSAC)            ← RANSAC.py
    │   homography(px1, px2)                   ← homography.py
    │   score_H(H, px1, px2)                   ← score.py
    │   decompose_H(H, K, plane_dist)           ← homography.py
    │
    ├── Branch F ─────────────────────────────────────────────────────
    │   RANSAC(s=8, score_F_RANSAC)            ← RANSAC.py
    │   eight_point(px1, px2)                  ← eight_point_agl.py
    │   score_F(F, px1, px2)                   ← score.py
    │   get_R_t_from_epipolar(F, K)            ← Retrieve_P.py
    │   parallax(P2_all, K, px1, px2)          ← Retrieve_P.py
    │   find_scaling_factor(...)               ← Retrieve_P.py
    │
    └── ratio = S_H / (S_H + S_F)  →  winner: H or F
    │
    ▼
_show_single(scene, res, cfg)          ← gui_tool.py
    ├── _draw_3d(...)                  ← visualize3d.py
    ├── _draw_image_cam1(...)          ← visualize3d.py
    ├── _draw_image_cam2_H(...)        ← visualize3d.py
    ├── _draw_image_cam2_F(...)        ← visualize3d.py
    └── _draw_score_bar(...)           ← visualize3d.py

## 5.3  File-by-file description

pipeline/core.py

Central orchestration file. Contains:

- Config — dataclass holding all parameter default values.

- make_scene(cfg) — generates the full synthetic scene (3D points → projection → noise → outliers) and returns a dict: {pts3d, px1, px2, K, R_rel, t_rel, R1, t1, urban_colors}.

- run_pipeline(scene, cfg) — orchestrates H and F branches, computes scores, makes the model decision, returns errors.

- run_noise_sweep(cfg) — loops make_scene + run_pipeline over each σ level to produce the noise sweep data.

pipeline/gui_tool.py

Full GUI built with customtkinter and matplotlib. Contains:

- App — main class.

- Reusable widgets: EntryRow, SliderRow, TriRow, _SectionTitle.

- _tab_scene / _tab_cameras / _tab_optics / _tab_analyse — tab construction functions.

- _collect_config() — reads all widgets and builds a Config object.

- on_run() — launches the pipeline in a background thread to keep the UI responsive.

- _show_single() / _show_noise() — update the matplotlib canvas after a Run.

pipeline/visualize3d.py

All matplotlib drawing functions. Contains:

- _draw_3d() — main 3D view: point cloud, frustums, camera axes.

- _draw_frustum() — draws one camera frustum (pyramid + directional arrow).

- _draw_cam_axes() — draws the three local RGB axes on a camera.

- _draw_image_cam1/2_H/F() — reprojection panels in the image plane.

- _draw_score_bar() — comparative histogram S_H vs S_F with threshold line.

- _urban_T() — coordinate transformation function for the Urban view.

pipeline/score.py

Scoring functions used after RANSAC for the final computation on inliers:

- score_H(H, px1, px2) — symmetric transfer error, threshold 5.99.

- score_F(F, px1, px2) — Sampson distance, threshold 3.84.

simulation/camera_model.py

- get_K(fx, fy, cx, cy) — builds the 3×3 intrinsic matrix K.

- get_camera_pose(rx, ry, rz, tx, ty, tz) — builds R and t from Euler angles and translation vector.

simulation/projection.py

- project_points(pts3d, K, R, t) — projects 3D world points to 2D pixels via P = K[R|t].

- filter_visible(px, depth, w, h) — boolean mask removing points outside the image bounds or behind the camera.

simulation/homography.py

- homography(pts_src, pts_dst) — normalised DLT homography estimation (SVD on 2N×9 matrix A), minimum 4 correspondences.

- decompose_H(H, K, plane_dist, X_ref) — extracts (R, t) from H. If plane_dist is known (planar scene), returns t in metres; otherwise returns a unit vector. Cheirality test via X_ref.

- reprojection_error_H() — RMSE reprojection error for H.

simulation/dlt.py

- dlt(points_2d, points_3d) — estimates the 3×4 projection matrix P by normalised Direct Linear Transform, minimum 6 2D↔3D correspondences.

- extract_Rt_from_P(P, K) — extracts R and t from P via SVD decomposition of K⁻¹P.

simulation/scene_urban.py

Generates the Urban synthetic scene:

- _sample_box() — uniformly samples points on the 6 faces of a box, proportionally to each face area.

- _sample_tree() — cylindrical trunk (lateral surface) + flattened hemispherical dome (spherical coordinates, φ ∈ [0, π/2]).

- _sample_car() — two stacked boxes (body + roof) via _sample_box.

- generate_urban_scene(rng, n_points) — assembles the three objects, distributes points (55%/25%/20%), returns (pts3d, colors).

eight_points/eight_point_agl.py

Normalised 8-point algorithm with optimisation:

- normalize(points) — isotropic normalisation (centring + scale).

- linear_eq(points1, points2) — builds the N×9 matrix A of the system Af = 0.

- eight_point(pts1, pts2) — full algorithm: normalise → SVD → rank-2 enforcement → Levenberg–Marquardt optimisation → denormalise.

- min_alg_error(F0, A) — minimises algebraic error.

- residual_fun(e, A, state) — cost functionn.

eight_points/RANSAC.py

- RANSAC class — adaptive RANSAC. Iteration count N is updated dynamically as inliers are found:

N = log(1−p) / log(1−(1−ε)^s), with p = 0.99.

- score_H_RANSAC / score_F_RANSAC — same logic as score.py, used during RANSAC to rank candidate models.

eight_points/Retrieve_P.py

- get_R_t_from_epipolar(F, K) — computes E = KᵀFK, decomposes E by SVD to get t and two rotation candidates R1, R2 via the W matrix.

- P_estimation(t, R1, R2, K) — assembles the 4 possible projection matrices K[R|±t].

- parallax(P2_all, K, pt1, pt2) — triangulates points for each solution and selects the one with the most points at positive depth (cheirality test).

- find_scaling_factor(P2, K, pts1, pts2, pts3D) — estimates the scale factor by comparing triangulated inter-point distances to ground truth distances.

eight_points/triangulation.py

- triangulate(P1, P2, pts1, pts2) — linear triangulation by SVD: for each point pair, builds the 4×4 DLT system and solves it to obtain the homogeneous 3D point.

# 6. Known Limitations

## Current limitations

- Synthetic data only. The tool works exclusively with generated scenes. No real image input or actual keypoint matching is implemented.

- Limited RANSAC control. The RANSAC toggle enables/disables outlier rejection, but inlier threshold and iteration count are not user-configurable from the GUI.

## Potential improvements

- Real image support — integrate actual image loading with ORB/SIFT feature extraction and matching to test on real stereo pairs.

- Advanced RANSAC tuning — expose inlier threshold and maximum iteration count as GUI sliders.

- Multi-view extension — support more than two cameras and incremental structure-from-motion.

- Export of results — allow exporting all metrics and estimated matrices to a JSON or CSV file for batch analysis.

Camera Pose Estimation Tool — Handoff & User Documentation — May 2026